MORTERO EQUIVALENTE AL HORMIGÓN AUTOCOMPACTANTE

MORTAR EQUIVALENT TO SELF-COMPACTING CONCRETE

Luis-Vicente García-Ballester

Departament Architecturals Constructions; Universitat Politècnica de València; Camino de Vera, s/n, 46022 Valencia

Senior Lecturer, PhD UPV. E-mail: lvgarcia@csa.upv.es

José Ramón Albiol-Ibáñez

Researcher in the Department of Architectural Constructions; Universitat Politècnica de València,  Assistant Professor, Materials Engineer, Building Engineer, Master in Concrete Engineering, PhD Engineering of the Construction and Environmental Management by the UPV.  Dr.h.C. by Odessa State Academy of Civil Engineering & Architecture. E-mail: joalib1@csa.upv.es

Miriam Carbonell- Córdoba

Technical Architect

Abstract

The concrete equivalent mortar to concrete (CEM) method consists of replacing the gravel which forms part of the concrete by a sand mass to provide a specific surface area equivalent to a mortar scale. This method can be used to reduce the time and cost required to design self-compacting concrete (SCC). The objective this research is to establish the concrete mortar equivalent to a certain self-compacting concrete and study the influence that the granulometry of the chosen sand can have and which may have the variation in the additive content. A reference SCC and several mixtures of CEM were mixed with three different sand granulometry and four proportions of superfluidific admixture. The results of the tests obtained show that there is a good correlation between the slump flow of the reference SCC and the CEM with the same percentages of admixture but that this property in the CEM is very sensitive to the amount of admixture, as it happens in the SCC. As with SCC, the behavior of the CEM is also affected by the grading of the sands and especially by the content of fines that these contribute.

Keywords

Concrete Equivalent Mortar, Self-Compacting Concrete, Specific Surface Area, Slump Flow